Econoseal 10 pinout TEMPLATE (although this might be the most frequently applied pinout, use the paper included with the device as reference). Backside of the female connector, or looking into the connector with male receptacles:

(always verify GND pin continuity to JPT55/pin10 or pin14, pin19, pin24, pin30)

1:	mcp3208 ch0 (protected analog input)

2:	mcp3208 ch1 (protected analog input)

3:	mcp3208 ch2 (protected and pulled up analog input)

4:	mcp3208 ch5 (protected and pulled up analog input)

5:	mcp3208 ch6 (pulled up analog input, Strictly 0-5V. Only if you know what you're doing, don't damage it !)

6:	2nd serial in (data from the TX of NMEA GPS „RS232” signal)

7:	p259 ch4 output (switching to GND ; primarily for LED with min 100 Ohm current-limiting resistor, or Check-engine-light or resistive load, max 300mA)

8: 	Ignition ch2 (IGBT)

9: 	Ignition ch3 (IGBT)

10:	GND

The serial cable is ready for Bluetooth or Wi-fi (+5V on DSUB9/pin9,   GND is DSUB9/pin 5)

JPT 55/5 Ignition ch7

JPT 55/9 mcp3208 ch7

Active coils (or power output stages) assumed, ignition outputs are driven by logic level ignition outputs (not suitable to directly drive passive-coils).