Dear Customer, Please KEEP THIS. READ CAREFULLY

VEMS GenBoard v3.x is very flexible, with room for customization. It is possible to get a working system relatively fast, but there are important things to consider even for an assembled, cased unit. Important Warnings - failing these will result in damage:

- Most ignition transformers have secondary winding GND return that MUST be grounded (to cylhead)

- Unplugging EC36 or EC18 while harness is powered up might cause serious ECU damage.

- connect all grounds 1 GND and 4 GND5-s from Econoseal-36 (GND5 is sometimes marked as powerGND or PGND) before applying power. GND and GND5 should meet 10 .. 15cm (5 inch) from the ECM. Use strong wires (0.75-1 mm2) and don't let any chance for disconnection during operation.

- there must be a fuse (max 10A, or 15A for multi-injector lowZ setups) between injector_common and +supply

- connect the flyback connection directly (no fuse) to the injector_common signal. There must be no fuse between injector_common and (EC36-pin 23) flyback pin.

- Flyback pin (EC36-pin 23) must be connected with airwire or 30V transient diode to onboard flyback trace. This must be done on the non-assembled unit (already done on the assembled controller).

- with DVM in diode mode, all injector outputs should measure max 1.4V between injector output (+) to injector_common (-) , (even with injectors disconnected). With the 30V flyback, reference (connect DVM-) to the internal flyback trace (verified for the assembled controller).

- use separate fuses for each ignition channel: 3-4A during testing, upgrade to 5A only after successful tests. 5A (RMS value!) fuse allows appr 9A peak current. Anything higher does not provide sufficient protection.

- use a 3-5A fuse for the WBO2 heater channel, and 1-3A fuse for the EC36/25 mainboard supply

- PWM-ing MUST be disabled (injector-pwm-duty=100%, injector full duty time=25.5msec). Use highZ injectors or lowZ with current-limiting "ballast" power-resistors connected in series.

Warning: Wheel speed in, Analog in ChX(except Ch1) is ment for 0-5V only. Connecting 12V might cause damage. Applying +12V to the input by connecting a solenoid, relay, injector or ignition coil might damage the controller. Switch these pins to EC36/26 GND. When no pullup inside, apply 2k4..4k7 pullup towards EC36/28 (+5V) for ALS/launch/shiftcut activation switch/button. “Analog in ChX“ is same as mcp3208 ChX (MCX). Protection is for minor static electricity (no misconnection!). Typical pinout:

** The only mcp3208 analog input where +12V can be applied via 2k7..4k7 (preferred use: on/off switching)

Order ID:				Serial Number:				Date:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (internal 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.

Order ID:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (internal 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.

Order ID:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (internal 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.

Order ID:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (internal 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.																	Order ID:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (optional 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.

Order ID:

EC18/13 WBO2 nernst: LSU4.2/pin1 or LSU4.9/pin6 (internal 27k nernstpullup to +5V, verify by DVM).

Disclaimer: this information is for convenience only, not enough for an install. Consider the webshop product page and web documentation before installing. Add your own notes.

Order ID:

Relay connection

“ignition” switched

+12V (node 15)

1N4007								Relay

- Preferrably ign IGBT output

- or injector output

- or stepper output

- (or low current “P259” output as a last chance)

Rescue1 kit

- 3x1n4007 1A throughole diode. Used across iac valve, relay, solenoid, cathode (stripe-marked) to +12V.

- 0805 SMD resistors: appr 15 values, 4..6 pieces of each. 104 (means 10*10^4) = 1003 (100*10^3)=100kOhm

- ES2J/ER2J 2A 600V fast SMB (surface mount, rather big) diodes (optional) Normally not needed

- optional: unidirectional transient suppression diode (black, bigger than 1n4007) 1k5E30 is 30V, 1k5E18 is 18V (across supply. Cathode to +12V). NOT to be used in the flyback path of a controller that already has 30V flyback internally

- red,yellow,black,brown,brown striped throughole resistor is 2.4kOhm. Pullup (or between LCD contrast and GND)

Rescue2 - capacitors and 2 zener diodes 
Capacitors don't have text on them, so -if ever needed- ways to tell their value:

- measure with DVM that can measure capacitance

- measure by applying 50..60 Hz sinewave with 10k limiting resistor and measure AC voltage across the resistor, than calc.

- keying by number, size (0603 not used, 0805 is small, 1206 is bigger), capacitors :

- Rescue2 v0.07:

- 3*1n 0805 50V

- 3*10n 0805 50V

- 4*100n 0805 50V

- 5*220n 0805 50V

- 2*1u 0805 25V (tall)

- 2*2.2u 0805 16V (tall)

- 1*10u 1206 25V

- 2*1u 1206 50V

- Update this printed list when using some capacitors, to keep track. www.vems.hu/wiki/ for more info