Dear Customer, Please DO NOT THROW THIS AWAY, READ CAREFULLY

VEMS GenBoard v3.x is very flexible, with room for customization. It is possible to get a working system relatively fast, but there are important things to consider even for an assembled, cased unit. Important Warnings - failing these will result in damage:

- connect all grounds 1 GND and 4 GND5-s from Econoseal-36 (GND5 is sometimes marked as powerGND or PGND) before applying power. GND and GND5 should meet 10 .. 15cm (5 inch) from the ECM. Use strong wires (0.75-1 mm2) and don't let any chance for disconnection during operation.

- there must be a fuse (usually 10A, or 15A for multi-injector lowZ setups) between injector_common and +supply

- connect the flyback connection directly (no fuse) to the injector_common signal. There must be no fuse between injector_common and (EC36-pin 23) flyback pin.

- Flyback pin (EC36-pin 23) must be connected with airwire or 30V transient diode to onboard flyback trace. This must be done on the non-assembled unit (already done on the assembled controller).

- with DVM in diode mode, all injector outputs should measure max 1.4V between injector output (+) to injector_common (-) , (even with injectors disconnected). With the 30V flyback, reference (connect DVM-) to the internal flyback trace (verified for the assembled controller).

- use separate fuses for each ignition channel: 3-4A during testing, upgrade to 5A only after successful tests. 5A (RMS value!) fuse allows appr 9A peak current. Anything higher does not provide sufficient protection.

- use a 3-5A fuse for the WBO2 heater channel, and 1-3A fuse for the board supply

- PWM-ing MUST be disabled (injector-pwm-duty=100%, injector full duty time=25.5msec. Use highZ injectors or lowZ with current-limiting "ballast" power-resistors connected in series.

Rescue1 kit

- 0805 SMD resistors: appr 15 values, 4..6 pieces of each. 104 (means 10*10^4) = 1003 (100*10^3)=100kOhm

- ES2J/ER2J 2A 600V fast SMB (surface mount, rather big) diodes (optional) Normally not needed

- 3x1n4007 1A throughole diode. Used across iac valve, relay, solenoid, cathode (stripe-marked) to +12V.

- optional: unidirectional transient suppression diode (black is , bigger than 1n4007) 1k5E30 is 30V, 1k5E18 is 18V (across supply. Cathode to +12V). Must NOT be put in the flyback path with controller that already has 30V flyback internally: 30V+30V would kill the FETs!

- red,yellow,black,brown,brown striped throughole resistor is 2.4kOhm. Pullup (or between LCD contrast and GND)

Warning: Wheel speed in, Analog in CHn is for 0-5V only. Connecting 12V causes serious damage. Connecting a solenoid, relay, injector or ignition coil kills the controller immediately because it applies +12V to the input. Switch these pins to EC36/26 GND. When no pullup inside, apply 2k4 pullup towards EC36/29 (+5V) for ALS/launch/shiftcut activation switch/button. Only experts may use EC36/28 for pullup, only for precise analog signals, like oilpressure sensor.

“Analog in Chn“ is same as mcp3208 Chn.Protection is for minor static electricity only(no misconnection!). Typical pinout:

Rescue2 - capacitors and 2 zener diodes 
Capacitors don't have text on them, so -if ever needed- ways to tell their value:

- measure with DVM that can measure capacitance

- measure by applying 50..60 Hz sinewave with 10k limiting resistor and measure AC voltage across the resistor, than calc.

- keying by size, number, tape-material examine the cap size (0805 is the smallest we use, 1206 is medium and 1210 is huge), the number of caps and the tape-material (trans is transparent plastic, paper is paper) will tell you the value according to a table below:

Rescue2 v0.07:

- 3*1n 0805 50V paper

- 5*10n 0805 50V paper

- 4*100n 0805 50V paper

- 6*220n 0805 50V paper

- 2*1u 0805 25V paper (tall)

- 2*2.2u 0805 16V trans (tall)

- 1*10u 1206 25V trans

- 2*1u 1206 50V trans

Update your printed list if you use some capacitors, so you can keep track. www.vems.hu/wiki/ for more info