# VEMS v3.3 Customer Guide

When installed correctly your VEMS ECU will provide you with long trouble-free service.  Please take time to read this guide, and pay special attention to the section on Ground Wires.  If you have any problems understanding any of the information here, please contact your VEMS Reseller.

The information contained here should be sufficient to install and run your pre-configured VEMS and wiring loom.  It is designed to compliment the VEMS User Guide which you should take time to read and refer to as you progress through the installation and calibration of your VEMS.  The guide is available on-line at:

http://www.vems.hu/wiki/index.php?page=MembersPage%2FPhatBob%2FUserGuide

## Unit Information

Your VEMS unit has some unique settings that you may need to refer to should you wish to upgrade the unit’s software.

Unit Serial Number:		xxx

Board Version:		v3.4

Wide band Settings:	pump-= x.xxV
wbo2_pump_pw_zero=0xXX (10X)

wbo2_calibration = 
wbo2_nernstdc_target=0xXX (13X)
pump+= -x.xmV/200 Ohm

AREF=x.xV

Primary Trigger:	x tooth

TDC after trigger:	x

Number of teeth:	x

Trigger tooth:	x

Next trigger tooth:	x

Crank min. period:	xuSec

Angular width of tooth:	x

Tooth rel. norm:	x

Tooth rel. miss:	x

Ang. width missing:	x

Eng. Phase reset:	x

Ignition:	Coil pack coil charge = x.xmS

MAP sensor:	xxxkPa

# Ground Wires

Bad grounding is the cause of 90% of noise and trigger errors with engine management systems, and bad grounding can damage your VEMS ECU.  Make sure that you read the follow section before you connect anything to

The engine block is the best point from many perspectives. But it is of some importance to keep this ground point away from the alternator and starter ground paths, you can do this but adding extra grounding wire at the alternator to ensure a path of low resistance.

Having the ground path for the alternator, starter and the ECU at the same point is not a good idea. So keep the ECU connection to the block as far away as possible.

- If the alternator is bolted to the engine block you way want to ground the ECU to the head

- If the alternator is bolted to the cylinder head you may want to ground the ECU through the engine block, well away from the ground strap.

- On engines like those fitted to some Audis (Where the alternator is bolted to the head) it is acceptable to bolt the ECU earth to the far side of the cylinder head.

Your ECU will always share some of the ground path with the alternator. You want to make sure that the shared path is as short as possible and that it is as low resistance as possible.

Grounding directly at the battery is not a good idea, at the negative of the battery you always see the current pulsations from the alternator and starter, the big currents flow from battery to starter and alternator to battery, although you may hope that the battery may act as a buffer, lead batteries are pretty slow and it doesn't filter the noise very well.

The placement of the battery also change a lot of things, if you have a battery located in the boot you could need up to five 4m wires. Also And the connection at battery connectors is often forgotten by techs, are subject to aggressive corrosion, abused when connecting jump leads.

Connecting to the chassis is often a bad idea; the body of the car is an inferno of ground currents. For example the wipers, electric seats (consuming 50A+ when operated) etc may be grounded at the same point as the ECU. And voltage fluctuations increase as you get further from the battery.

# Power Connections

The loom requires two power feeds, the first needs to be connected from the car’s main fuse to relay pin 87.  A second connection needs to be made from a switched supply to relay pin 85.  Relay pin 86 must be connected to ground.

The loom has been supplied with only one fuse in place, a 1A fuse for the VEMS unit.  It is not advisable to connect any of the other fuses until you have tested that the VEMS is powered correctly, and that it is able to read the crank sensor.

# Testing VEMS’ power supply

Connect the Two Econoseal plugs to the VEMS unit.

With VEMS plugged in and wired as directed above, connect your laptop to the loom’s serial connector.  Start the MegaTune program and switch the ignition on.  You should see the gauges on the program jump around

# Testing the Crank Sensor

Connect the crank sensor to the junior timer plug on the thick cable.  Turn on the ignition and crank the engine over, you should see the RPM gauge read approx 300rpm.

Turn off the ignition.

Connect up the rest of the sensors, Air and water temp sensors and the air pressure sensors.  Once you have done this turn the ignition back on and you should see the gauges giving proper readings.

# Battery Calibration

Measure the VEMS supply voltage with a DVM.

Menu option Settings->Constants

Adjust the Battery Calibration value until the gauge on the main screen displays the same voltage as the DVM

The calibration is usually performed as part of the final test process, but it is worth checking.

# Configure Throttle Position Sensor (TPS)

Closed throttle

Make sure that the TPS is fastened correctly and that the throttle is closed.

Menu option Settings->Basic Settings

With your foot off the throttle change the TPS Low (ADC) value until the Throttle Position gauge on the main screen reads 1% and then decrease the TPS Low value by one.

Wide Open Throttle (WOT)

Make sure that the throttle is fully open when the accelerator pedal is fully down.

If you adjust the idle screw you may need to repeat this process.  Moving the TPS will require recalibration.

# Configure MAP Sensor

For super accurate calibration you might want to consult your local weather forecast, and check the current air pressure, which will be between 999 and 1030 millibars.

Menu option Settings->Basic Settings

Make sure that the Map Sensor range (kPa) value matches that of the sensor that you are using.

Adjust the Map Sensor offset (kPa) so that the Engine MAP gauge on the front screen.

The MAP sensor calibration is usually performed as part of the final test and configuration process.

# Ignition

Menu option Settings->Priming, Cranking, Afterstart

Check that Crank Advance is 10.00

Insert the ignition fuse, and check that the coil pack is connected.

Connect your Timing light on coil 2 and make sure that the plug is earthed.

Crank the engine and check that the timing mark is correct.

If you cannot see the timing mark you may be 180 degrees out, so its worth trying the timing light on the other coilpack.

If the mark is not exactly on 10degrees:

Menu option Settings->Primary Trigger

Adjust the TDC after the trigger (deg) value and crank the engine again.

With a good battery and crank pulley trigger you should see perfect timing alignment.

# Fuel Pump

VEMS controls the fuel pump relay from a single wire on the 36pin Econoseal connector

# Wideband

The wideband has been configured; it requires a 5Amp fuse to be fitted to its fuse holder.

# Injectors

This system has been configured for high resistance injectors, ensure that the injector’s resistance is more than 12Ohms. If they are lower resistance then the system must be configured differently.  Otherwise connect the fuel injector 10A fuse, start the engine and drive away!

# Installation Notes

In order to protect the system from boosting into an un-mapped region Fuelcut above kPa (Settings->Basic Settings) has been set to 120kpa, when tuning the boost controller the Fuelcut will need to be increased by degrees as the system is mapped and the controller tuned.

# Upgrading Firmware

As new features become available the software that runs on the VEMS hardware can be upgraded.  Some small programs have been provided to enable this, the programs are listed in order of use.

- Download-config.bat takes the configuration and map tables from the VEMS unit and saves them into the config directory.

- Generate-config.bat uses the downloaded config and tables files and converts them into a format that can be uploaded to the VEMS.

- Upload-firmware.bat saves the generated config and table files to the VEMS unit.