EC type / VEMS pin / Motronic55 Pin Function / AssignmentInput or Output wrt ECU Schematic References Wire Colour at ECU

X  = not connected/not avaiable

18 / number = connected into VEMS EC18/number ,all other numbers int he first row means VEMS EC 36 connector!!!

36 / 11 / 01 Power output stage for ignition, cylinder #1OUTPUT N122 pin I1 gn/ws

36 / 5 / 02 Power output stage for ignition, cylinder #2 OUTPUT N122 pin I2 li

18 / 03 Fuel Pump Relay, 0V. Also provides 12V power for various components.OUTPUT (0V) J17 br/gn

36 / 6 / 04 Idle Air Control Valve, 0V OUTPUT (0V) N71 pin 2 gn/sw

X / 05 Evaporative emissions frequency valve, 0V OUTPUT (0V) N80 pin 1 ws

X / 06 Air conditioning compressor control. Normally an input in conjuction with pin 41 , but can also be     used    to turn off A/C /

compressor when maximum engine power is required INPUT / OUTPUT A/C gn/sw

X / 07  Mass Air Flow (MAF) sensor signal INPUTG70 pin 3 sw/ws

36 / 13 / 08  Camshaft Position Sensor (Hall Sender), Pin #2 INPUT G40 pin 2 gn

X / 09 Barometric pressure sensor INPUT F96 pin 1 gr

36 / 26 / 10 0V ground/shield for various sensors 0V G39, G70, G4, G28, G61, G66, F60, G40 br/ge

X / 11 Knock sensor I (For cylinders 1 - 3) INPUT G61 ws

36 / 28 / 12 +5V reference voltage for various sensors OUTPUT G69 pin 1, G40 pin 1, F96 pin2 ro/sw

X / 13 OBD connector (L-wire) INPUT OBD ws/ro

GND / 14 0V ground 0V - br/ro

X / 15 Not Used

36 / 9 / 16 Fuel injector -ve pulse, cylinder #5 OUTPUT N83 pin 2 br/ro

36 / 19 / 17 Fuel injector -ve pulse, cylinder #2 OUTPUT N31 pin 2 br/bl

X / 18 Permanent 12V supply voltage 12V INPUT S27 ro/ws

36 / 26 / 19 0V ground/shield for various sensors 0V G39, G70, G4, G28, G61, G66, F60, G40 br/ge

18 / 10  / 20 Power output stage for ignition, cylinder #4 OUTPUT N127 pin I1 ro/sw

18 / 4  / 21 Power output stage for ignition, cylinder #5 OUTPUT N127 pin I2 gn

36 / 22 OBD for Malfunction Indicator Lamp (MIL) - Blink Codes OUTPUT OBD gr/br

36 / 11 / 23 Power output stage for ignition, cylinder #3 OUTPUT N122 pin I3 sw/bl

GND / 24 0V ground 0V - br/ro

X / 25 Mass Air Flow (MAF) sensor, burn-off signal OUTPUT G70 pin 4 bl/li

X / 26 Mass Air Flow (MAF) sensor, electrical ground INPUT G70 pin 2 sw/gn

36 / 25 / 27 Switched 12V supply voltage (Ignition ON) 12V INPUT S32 sw

X / 28 Oxygen / Lambda sensor INPUT G39 gn

X / 29 Knock sensor I (For cylinders 4, 5) INPUT G66 gn

GND / 30 0V ground 0V G61, G66. G69, F96, G62, G42 sw/li

X / 31 Fuel consumption signal OUTPUT J128/189 br/sw

X / 32 Charge pressure signal (NOT used on the S2) OUTPUT

36 / 17 / 33 Wastegate frequency valve control signal (Switched 0V) OUTPUT N75 pin 2 gn/ge

36 / 8 / 34 Fuel injector -ve pulse, cylinder #3 OUTPUT N32 pin 2 br/gn

36 / 20 / 35 Fuel injector -ve pulse, cylinder #4 OUTPUT N33 pin 2 br/ws

36 / 7 / 36 Fuel injector -ve pulse, cylinder #1 OUTPUT N30 pin 2 br/sw

36 / 23 / 37 Switched 12V supply voltage (via fuel pump relay - activated by ECU). Also related to 'holdover relay' with ECU for the MAF. Probably for burnoff - assuming that pin 25 burnoff trigger is a 0V output pulse. In which case this pin is pulled to 12V with the engine running and is used to empower the MAF burnoff sequence with the engine off.

12V N30, N31, N32, N33, N83, G70 bl/sw

X / 38 Coding Plug (where applicable) INPUT - br/bl

X / 39 Coding Plug (where applicable) INPUT - ws/sw

4 / 40 Engine speed signal (RPM) OUTPUT G5 (Tacho) li

X / 41 Air conditioning status - informs the ECU if A/C is ON or OFF INPUT A/C sw/ge

X / 42 Not Used

X / 43 Not Used

36 / 2 / 44 Intake air temperature INPUT G42 sw/ge

36 / 14 / 45 Coolant temperature INPUT G62 ro/gn

X / 46 Multi-Function Thermal Switch INPUT F76 pin 2 (R) bl/ws

18 / 12 / 47 Crankshaft reference position sensor (62 degrees BTDC on cylinder #1) INPUT G4 ro

36 / 26 / 48 0V reference for crankshaft sensors. 0V G4, G28 li-G4 gr-G28

36 / 27 / 49 Crankshaft speed sensor (135 teeth on flywheel starter ring) INPUT G28bl

X / 50 Vehicle speed signal (VSS from speedometer) INPUT G21 ws/bl

X / 51 Not Used

X / 52 Throttle position switch (Idle detection) INPUT F60 pin 6 gn/li

36 / 1 / 53 Throttle position potentiometer (Throttle angle) INPUT G69 pin 3gn/bl

X / 54 Not Used

X / 55 OBD connector (K-wire) INPUT / OUTPUT OBD gn/ro